# Wren’s Nursery Electrical Safety and Testing Policy

## 1. Purpose

This policy ensures that all electrical systems and equipment at Hooke Court and Wren’s Nursery are maintained in a safe condition, preventing risk of electric shock, burns, fire and equipment failure, in accordance with current UK legislation and HSE guidance.

## 2. Legislative Framework

This policy complies with the following legislation and guidance:

- Electricity at Work Regulations 1989

- Health and Safety at Work etc. Act 1974

- Management of Health and Safety at Work Regulations 1999

- BS 7671 – IET Wiring Regulations (18th Edition)

- HSE Guidance Note HSG107 – Maintaining Portable Electrical Equipment

- HSE INDG236 – Maintaining Portable Electrical Equipment in Low-risk Environments

## 3. Fixed Electrical Installation Testing

All mains electrical installations must be inspected and tested by a competent electrician in accordance with BS 7671.

| Area Type | Test Interval |
| --- | --- |
| Educational premises (schools, nurseries, training centres) | Every 5 years |
| High-risk or heavily used areas | As advised by the electrician |

- A Electrical Installation Condition Report (EICR) must be produced after each inspection.

- Copies of certificates and wiring diagrams must be stored in the main office.

- Any C1 or C2 defects must be rectified immediately.

## 4. Portable Appliance Safety (PAT)

Hooke Court follows the HSE’s risk-based approach to Portable Appliance Testing rather than fixed annual testing.

Electrical equipment is controlled through three levels of inspection:

- User visual checks

- Formal visual inspections

- Combined inspection and testing (PAT)

The level and frequency depend on:

- Equipment type

- How often it is used

- Who uses it

- The environment in which it is used

## 5. User Visual Checks (Daily Use)

Every time an item of electrical equipment is used, staff must carry out a quick visual check.

The equipment must not be used if any of the following are observed:

- The equipment is not in good working order

- The cable sheath is cut, frayed or damaged

- The plug is cracked, loose or has bent pins

- Internal coloured wires are visible

- The socket is overloaded or extension leads are daisy-chained

- The equipment is wet or contaminated

- The casing is cracked or screws are missing

- There are scorch marks or signs of overheating

- Extension leads create a tripping hazard

Unsafe equipment must be:

- Switched off

- Unplugged

- Labelled “DO NOT USE”

- Reported to the site manager immediately

## 6. Formal Visual Inspections

A nominated trained person will carry out recorded visual inspections of electrical equipment.

These checks will look for:

- Damage to plugs, cables and casings

- Signs of overheating

- Loose or exposed wiring

- Missing or incorrect fuses

- Evidence of misuse

Records must include:

- Equipment ID

- Location

- Date of inspection

- Inspector’s name

- Action taken

Records must be kept for at least 4 years.

## 7. Portable Appliance Testing (PAT)

PAT testing will be carried out by a competent person or contractor.

Typical inspection intervals:

| Equipment Type | Typical Test Frequency |
| --- | --- |
| IT equipment (PCs, printers, monitors) | Every 2–4 years |
| Class I equipment (kettles, heaters, microwaves) | 1–2 years |
| Extension leads and power strips | 1 year |
| Hand-held or frequently moved equipment | 1 year |
| Low-risk, fixed equipment | Visual inspection only unless risk increases |

Test labels must show:

- Date tested

- Pass or fail

- Tester’s initials

Failed equipment must be removed from service immediately.

## 8. New, Hired or Personal Equipment

- All new equipment must be visually checked before use.

- Hired equipment must arrive with a current PAT certificate.

- Staff bringing personal electrical items (e.g. fans, chargers, heaters) must have them approved and tested before use.

## 9. Training and Responsibilities

All staff are responsible for:

- Carrying out visual checks before use

- Reporting faults immediately

- Never attempting repairs unless authorised

The Site Manager / Health & Safety Lead is responsible for:

- Arranging inspections and PAT testing

- Keeping records

- Ensuring defective equipment is removed

## 10. Fire and Emergency Safety

If electrical equipment overheats, sparks, smokes or smells:

- Switch off immediately if safe

- Unplug the equipment

- Evacuate if necessary

- Report to management

Only CO₂ or dry powder extinguishers may be used on electrical fires.